Pass Through Print/Initialization Codes

last revision:  02/14/01  - jlf

IBM Proprinter Codes

IBM Proprinter 10 CPI Utility	H1B46121B461B48

IBM Proprinter 17 CPI Utility	H121B460F1B461B48

HP PCL 5 Codes (Lexmark, HP LaserJet 5 or newer, all HP DeskJets)

HP LaserJet/DeskJet 10 CPI	H1B266B3053

HP LaserJet/DeskJet 17 CPI	H1B266B3253

HP LaserJet 6 LPI	H1B266C3644

HP LaserJet 8 LPI	H1B266C3844

HP PCL II Codes (Older HP LaserJets)

HP Laser I-IV 10 CPI	H1B2873313048

HP Laser I-IV 17 CPI	H1B2873313748

All other Impact Printers	use IBM Proprinter codes

All other LaserJet Printers	use HP Laser codes

HP Initialization Codes on the Web

http://www.hp.com/cposupport/printers/support_doc/bpl02705.html

http://www.hp.com/cposupport/printers/support_doc/bpl02703.html

Other HP Codes

HP Auto CR	H1B266B3347

HP Auto LF	H1B266B3147

HP Landscape	H1B266C314F

HP Portrait	H1B266C304F

HP Reset	H1B69

HP Tray 1	H1B266C3848

Paper Cassette / tray 2	H1B266C3148

Manual Feed Paper	H1B266C3248

Manual Feed Envelope	H1B266C3348

MP Tray/ Tray 3	H1B266C3448

Optional Lower Cassette	H1B266C3548

HP Laser I-IV XY (hex) CPI	H1B2873XXYY48

Other Printer Codes

IBM Proprinter 10 CPI NLQ	H1B4512

IBM Proprinter 12 CPI NLQ	H1B451B3A

IBM Proprinter 12 CPI Utility	H1B461B3A1B461B48

HP LaserJet/DeskJet 12 CPI	H1B266B3453

HP Laser I-IV 12 CPI	H1B2873313248

Canon BubbleJet 17cpi	H1B0F

Canon BJC-5100 	As of 2/23/00 all BJC Windows-only

Canon S450	As of 12/05/00 Windows Only

Citizen 145GSX 10 CPI	H120000

Citizen 145GSX 17 CPI	H0F0000

Epson 6 LPI	H1B32

Epson 8 LPI	H1B30

Epson Courier Font	H1B6B02

Epson 10 CPI	H1B50

Epson 12 CPI	H1B4D

Epson 15 CPI	H1B67

Epson Condensed On	H1B0F

Epson Condensed Off	H1B12

Epson codes at http://www.epson.com/support/instans/esccodes.html

Lexmark Laser Tray 1	H1B266C3148

Lexmark Laser Tray 2	H1B266C3548

Lexmark Laser Tray 3	H1B266C3448

Lexmark Laser Aux Feed	H1B266C3348

Okidata Printers in Microline Emulation

Oki 1/2/3xx 10 CPI Utility	H1B301E

Oki 1/2/3xx 12 CPI Utility	H1B301C

Oki 1/2/3xx 17 CPI Utility	H1B301D

Oki 1/2/3xx 10 CPI NLQ	H1B311E

Oki 1/2/3xx 12 CPI NLQ	H1B311C

Oki 1/2/3xx 17 CPI NLQ	H1B311D

*** NOTES ***

1) Okidata Printers:  set the emulation to IBM PPR if available.

If not, use Epson emulation/codes.

2) On Panasonic printers, in order to set 8 lines per inch (LPI)

use the hex code: "1B30"

3) "Windows Only" printers are not suitable for passthrough

printing with U/Gi 3.10

4) For Windows NT set the following in UTFMCQ:

Form Feed Flag:        F (Flush print)

Only Open Exclusively: O

5) Please setup both 10 and 17cpi options in UTFMCQ

6) The old gn.printer_cds is in /u/home/james/wp

7) *** There are several printers that do not support MS-DOS printing. If the printer does not suport MS-DOS printing, then we will not be able to pass-through print through U/Gi.

Pass Through Print Codes

Terminal	Decimal	Hex

ANSI	PTP or %

Wyse 50	%001001024001001020	&180101140101

Wyse 60/150/160	%027100035020001001	&1B6423140101

Back to screen	%032066066069069

DG-D215	%030070096070097