Bekins Direct-Connect Setup

Last Revision: 11/13/98 - cjh

The following are setup notes taken from the Bekins Direct Connect Installation at Far West Moving Systems during 11/2/98 - 11/13/98. - Some of the settings May be specific to their site, and others could be general to all installations such as this.

At the time of installation, their network consisted of a Unix Server running SCO Enterprise, PCs with 10baseT Network cards, two Cisco 1500 Micro Hubs and a Cisco 1600 Router. - Geologistics (Division of Bekins Van Lines) provided the Router (already setup), both hubs and one HP computer running Windows 95 (later to be used for the warehousing system).

Geologistics also supplied the following software:

Microsoft SNA server, Outlook97, pcANYWHERE and Document Direct32. - These were not licensed individually, but were all installed on each PC.

Contacts at GeoLogistics:

David Aubert -	(708) 547-2225

Network Engineer, on approval by Kathy, contact Dave with any technical issues regarding installation of hardware or software including pcANYWHERE, Microsoft Outlook, Document Direct32 or Microsoft SNA Server. He is the primary technical support person for the Direct Connect installations.

Brad  Kamberos -  (708) 547-3456

Becom/Unix Direct Connect Interface - Contact Brad with any issues regarding installation of BECOM (Old Bekins Interface), Eicon card issues or issues regarding the Direct connect interface to the unix server.

Network Settings:

Required Installed Components:

Client for Microsoft Networks

In the installation at Far West, they did not have their own Windows NT server upon initial installation, so we needed to set this option to Log on to the Windows NT Domain of "Bekins1".

File and printer sharing for Microsoft Networks

Network Card Device

TCP/IP

TCP/IP Settings:

Bindings

"Client for Microsoft Networks"

"File and printer sharing for Microsoft Networks"

DNS Configuration

Enable DNS

Host: Windows Computer Name (found in Identification tab of Network Settings) - Also should be the same as the Server logon Name. At Far West, an example name is "Agent0489-3"

Domain: Bekins1

DNS Servers: (one only) - 192.168.0.14

Gateway: (address of router) ex. 192.168.47.1

WINS Configuration

Enable WINS Resolution

Server: 192.168.0.14

IP Address: Defined by Geologistics, in the case of Far West, IP addresses were in the range of 192.168.47.20 - 192.168.47.2x with a Subnet mask of 255.255.255.0

Far West IP Address Configuration:

192.168.47.1	Router

192.168.47.10	Unix Computer

192.168.47.180	IBM Network Printer

192.168.47.2x	Workstations

Additional Software configuration:

Microsoft SNA Server

From the SNA directory on the Bekins CD, run Setup. Take the defaults on each option until the Select Components Window comes up. - Select the following:

3270 Applet

ODBC-DRDA Driver (default)

AFTP Client (default)

Select the TCP/IP option in the "Select Client/Server Protocols" window.

Select "Client locates servers by name" in the "Client Mode window.

In the "Remote Server Names" Window enter "BVLCORP-SNA" for Primary.

At this point instalation will be completed, and you will need to create a shortcut of the 3270 Applet on the desktop. Once you run the 3270 Applet, it should bring up the SNA Server logon window, and this should be changed to logon to the NT domain. Once this is setup and working, set the "Autcconnect" option in the Session menu of the 3270 Applet.

Per Geologistics request, access the Session Configuration, and find the session name NSRVxxxx. - This will be different on each PC. Rename the 3270 Applet icon to have this name. - This is done for Bekins support purposes.

Document Direct 32

From the Bekins CD, drag the Docdirect folder to the appropriate drive on the PC. once this is done, open the DDR folder and locate the RDSWIN.EXE file, and create a shortcut on the desktop.

pcANYWHERE

Run the "Cdinst" icon from the Bekins CD in the pcANYWHERE directory.

Microsoft Outlook 97

Install Outlook by running Setup and taking the default options.

Once installed, open the Control Panel, and run the "Mail" or "Mail and Fax" icon. Will need to Add a profile using the "Microsoft Exchange Server" option. The Server name will be "BVLEXCHANGE", and the mailbox will the the agent logon (for Far West, mailbox is "Agent0489" for all workstations.)

** Also need to modify the unix Menus and add the following option:

GeoLogistics (Mainframe Download)

cd /u/bekins; download